RAIL SIMULATOR

Technical Guide

Methods, Formulas, Parameters, Reference Tables and Standards

v1.2  --  Mohamed BOUDIA

SCOPE This guide documents the physical models, calibration data, formulas and engineering standards behind the Rail Simulator. It is structured in three parts: - Part A -- Rail physics: wear, RCF, speed and grade effects - Part B -- Maintenance: grinding, reprofiling, tamping - Part C -- Calibration, validation and limitations

PART A

How to Use the Simulator

# A1. Introduction and Scope

The Rail Simulator estimates rail wear progression, grinding cycles, reprofiling interventions, replacement timelines and full lifecycle costs for tram, metro/LRT and heavy rail. It is intended for engineering studies, maintenance strategy comparison, lifecycle planning and budget preparation. It is NOT intended to replace local maintenance standards, safety acceptance, formal certification or detailed vehicle-track dynamic studies.

## A1.1 What the Simulator Covers

| Module | Description |
| --- | --- |
| Vertical wear (wV) | Crown wear progression on the running surface, mm/100MGT |
| Lateral wear (wL) | Gauge-face wear in curves, mm/100MGT |
| RCF index | Rolling contact fatigue accumulation and crack initiation |
| Preventive grinding | MGT-scheduled, crown only, 0.20 mm/pass |
| Corrective grinding | Condition-based (wV or RCF threshold), up to 4 passes |
| Reprofiling | Geometry-triggered, crown + gauge face, radius-based removal |
| Metal reserve | Vertical and lateral reserve tracking -- prevents grinding beyond physical limit |
| Replacement logic | Triggered by wV, wL, reserve exhaustion or RCF >= 0.70 |
| Grinding stone consumption | By machine type, radius band, rail grade |
| Cost models | Replacement, grinding, reprofiling, tamping -- 6 regional presets |
| Validation | Live comparison against 5 reference cases with back-calculated base rates |
| Ballast tamping | Preventive schedule, top-up, degarnissage -- ballasted track only |
| Corrugation | Simplified peak-to-peak amplitude state on special zones |

## A1.2 What the Simulator Does Not Model

- Full wheel-rail contact mechanics problem (no Hertzian contact solver)

- Full wavelength spectrum of corrugation (no RMS roughness law, no dynamic instability solver)

- Inner/outer rail asymmetry explicitly: outer rail is always the critical side. Inner rail is approximately 30-60% of outer rate.

- Wheel profile evolution over time: contact geometry is assumed constant

- Short-term transient sequencing within a year: annual time step smooths intra-year events

- Switch and crossing wear: different mechanisms, out of scope

- Seasonal variation: autumn leaf fall (+15-25% wear), winter ice (altered friction mode) not modelled

- Rail inclination/canting effects on contact geometry

- Temperature effects on rail steel properties

# A2. How to Use the Simulator

The interface follows a parameter-first philosophy: the left panel contains the project definition, while the right panel contains simulation results and cost tabs. In practice, the physical assumptions should be defined first and the outputs should only be interpreted afterwards.

## A2.1 Recommended Setup Workflow

| Step | Action | Key parameters |
| --- | --- | --- |
| 1 | Define traffic | Passes/day, axle load, fleet composition -- check Equivalent MGT vs Gross MGT |
| 2 | Define radius bands | Representative radius for each track segment, activate/deactivate bands |
| 3 | Add special zones | Stations, braking zones, corrugation-risk areas -- set local speed and fVExtra |
| 4 | Set rail parameters | Rail type, profile (limits and reserves), steel grade, track form |
| 5 | Choose strategy | Preventive (MGT interval) or Corrective (condition-based) |
| 6 | Configure limits | Use profile defaults or manual override for V and L limits |
| 7 | Configure reserves | Profile defaults or manual override; enable reprofiling if relevant |
| 8 | Run simulation | Click Run -- check for errors before reading results |
| 9 | Interpret results | Follow the interpretation order below |

## A2.2 Recommended Interpretation Order

| Priority | Tab / Chart | What to check | Why |
| --- | --- | --- | --- |
| 1 | Wear V and L charts | Wear trajectory per segment -- shape of curve, rate per band | Verify physics are coherent before reading costs |
| 2 | RCF Index chart | RCF evolution -- does it reach 0.70 before wear limit? | Understand whether fatigue or wear drives replacement |
| 3 | Metal Reserve chart | V and L reserve depletion -- which is the binding constraint? | Identifies reserve-limited vs wear-limited segments |
| 4 | Schedule chart | When grinding, reprofiling and replacement fire | Cross-check trigger logic and maintenance frequency |
| 5 | Grinding Cost tab | Total grinding cost per segment over horizon | Only meaningful if schedule is physically coherent |
| 6 | Reprofiling Cost tab | Total reprofiling cost over horizon | Check mobilisation vs operation breakdown |
| 7 | Replacement Cost tab | Rail renewal cost per segment | Verify unit costs against regional presets |
| 8 | Strategy Comparison | Preventive vs Corrective delta in lifecycle cost | Final decision support metric |
| 9 | Validation panel | Delta % vs reference cases BE1-GZ1 | Must be <30% before using results for budget decisions |

Note: A cost result is only meaningful if the triggering physical behaviour is coherent. Never read the cost tabs before verifying the wear, RCF and reserve charts are producing physically plausible trajectories for your context.

# A3. Model Architecture and Simulation Loop

The simulator is annual and segment-based. Each segment carries its own state variables and is updated independently each year. Standard radius-band segments and special zones are both converted into simulation segments and treated through the same core engine.

## A3.1 State Variables per Segment

| State variable | Symbol | Description | Reset by |
| --- | --- | --- | --- |
| Vertical wear | wV | Crown loss in mm (accumulates) | Grinding (-rem mm), Reprofiling (-reprRemV) |
| Lateral wear | wL | Gauge-face loss in mm (accumulates) | Reprofiling only (-reprRemL) |
| RCF index | rcf | Normalized fatigue damage 0-1 (accumulates) | Grinding (rcfReduction), Reprofiling (reprRcfR) |
| Vertical reserve | res | Metal available for grinding, mm (decreases) | Consumed by grinding and reprofiling |
| Lateral reserve | resL | Metal available for reprofiling, mm (decreases) | Consumed by reprofiling only |
| MGT since grinding | mgtSG | Accumulates each year; reset to 0 after grinding | Grinding event or reprofiling skip |
| Post-grind wear factor | pgLeft | MGT remaining at reduced wear rate after grinding | Decreases each year by mgtPY |
| Corrugation amplitude | corrAmp | Peak-to-peak amplitude mm (special zones only) | Partial reset after grinding |

## A3.2 Annual Simulation Loop (per segment, per year)

| Order | Operation | Key logic |
| --- | --- | --- |
| 1 | Update accumulated MGT | totMGT += mgtPY; mgtSG += mgtPY |
| 2 | Apply wear increment | wV += (mgtPY/100) x wrV x pgFactor; wL += (mgtPY/100) x wrL x pgFactor |
| 3 | Update RCF | rcf += rcfBase x mgtPY x (1 - crackArrestFactor) |
| 4 | Update corrugation | corrAmp += corrGrowth x mgtPY (if corrugation active on zone) |
| 5 | Evaluate reprofiling | Trigger if wL >= reprThreshFrac x limitL AND reserves sufficient |
| 6 | Evaluate grinding | Fire if any of 5 trigger conditions met AND reserve OK AND rcf < RCF_MAX |
| 7 | Check replacement | Replace if wV >= limitV OR wL >= limitL OR res <= minRes OR rcf >= 0.70 |
| 8 | Store yearly record | Push {year, mgt, wearV, wearL, rcf, res, resL, ground, reprofiled, repl} to data[] |

Important: ignoring same-year grinding or reprofiling on the replacement year is a display/reporting override only. It does not modify the core engine simulation sequence -- the physical calculation is preserved.

## A3.3 Physics Engine Summary

| Component | Model | Reference | Key formula |
| --- | --- | --- | --- |
| Wear (vertical) | Archard wear law with Eisenmann dynamic load | Infrabel/TU Delft 2023 | wrV = 0.82 x f_V x he x f_type x f_track x f_speed |
| Wear (lateral) | Archard wear law with flange contact multiplier | Infrabel/TU Delft 2023 | wrL = 1.00 x 1.5 x f_L x he x f_type x f_track x f_speed x f_lubr |
| RCF | Cumulative damage ratio with shakedown logic | Ringsberg 2001, IHHA 2019 | rcfBase = rcfRate[band] x f_grade x f_speed x f_lubr_rcf |
| Corrugation | Simplified peak-to-peak amplitude state | Grassie 2005 | corrAmp += growth x MGT; reset to trigger x resetFrac after grind |
| Tamping | MGT interval model with platform quality | ORE B17, RATP/STIB REX | Interval = BASE x f_platform x f_speed |

Note: The calibration baseline is: 0.82 mm/100MGT vertical wear, R260 grade, tangent track, ballasted, 80 km/h. Source: Infrabel/TU Delft 2023 big-data study (5338 km, Belgium 2012-2019) supplemented by Guangzhou Metro field measurements (2021-2022).

# A4. Rail System Parameters -- Roles and Interactions

The main parameter families are not interchangeable: each controls a distinct physical mechanism. Confusing their roles leads directly to incorrect calibration.

## A4.1 Rail Parameters -- What Each Family Controls

Rail type and rail profile define the geometric section and the default dimensional envelope of the rail. Rail grade defines the resistance of the material. Track form, speed and lubrication define the operational severity of the wheel-rail interaction. Maintenance strategy defines how damage is removed over time. These families must therefore be calibrated separately.

Current implementation hierarchy: rail profile provides default wear limits and reserves; manual overrides, when activated, take priority. This is important because the simulator no longer uses purely context-based limits as its primary reference.

| Family | Parameters | Controls | Effect if wrong |
| --- | --- | --- | --- |
| Geometry | Rail type, Track form, Radius band | Contact regime -- how the wheel meets the rail | Wrong contact type -> wrong wear mode (rolling vs sliding) |
| Material | Rail grade (BHN), Steel hardness | Damage resistance -- how fast the material degrades | Wrong grade -> wrong wear rate and RCF susceptibility |
| Maintenance | Grinding strategy, interval, passes; Reprofiling threshold; Tamping | Recovery rate -- how often and how much damage is removed | Wrong strategy -> wrong lifecycle length and wrong cost |
| Limits | Wear limits V/L, Metal reserves V/L, Min reserve thresholds | End-of-life -- when replacement is triggered | Wrong limits -> wrong replacement year, wrong total cost |

## A4.2 Special Zones -- When and Why to Use Them

The standard radius-band model assigns a single wear rate to each band across the entire line. Special zones allow localised overrides for areas where this is insufficient:

| Zone type | Typical location | Physical effect | Parameters to set |
| --- | --- | --- | --- |
| Braking zone | Station approaches | High V wear from wheel slip; corrugation risk | fVExtra, corrugation band, speed |
| Acceleration zone | Station departures | High V wear from wheel spin; corrugation risk | fVExtra, corrugation band |
| Station/terminus | Platform sections | Very low speed; high dwell time; creep forces | Speed override, railGrade |
| Transition | Curve-to-straight transitions | Mixed wear mode; geometric defects accumulate | Radius, speed |
| Corrugation-enabled zone | Any braking / acceleration / terminus / transition zone | Local roughness growth with grinding trigger on p-p amplitude | corrBand, corrAmp, corrTrigger, corrMGT fallback |

Corrugation on special zones is tracked as a simplified peak-to-peak amplitude state. When amplitude crosses the trigger threshold, grinding fires (1 pass if corrugation is the only trigger). The event is included in all cost tabs and the strategy comparison panel.

Recommended default wavelength bands in the current simulator are 30-100 mm for braking, acceleration and terminus areas, and 100-300 mm for transition zones.

If no measured roughness exists, the current p-p amplitude should start at 0.000 mm for a clean zone and only be set close to the trigger if grinding is already due from observed corrugation.

The corrugation MGT field remains a legacy fallback. The preferred trigger is now the simulated peak-to-peak amplitude crossing the selected threshold; corrugation-only corrective grinding is limited to one pass by design.

# A5. Maintenance Strategies -- Summary Overview

The simulator implements four maintenance operations. They interact through shared metal reserve consumption and through the skip-next-grinding logic. Understanding these interactions is essential for interpreting lifecycle results.

| Strategy | Trigger | Metal removal | RCF reset | Post-op protection | Best for |
| --- | --- | --- | --- | --- | --- |
| Preventive grinding | MGT interval reached (band x context); special zones may also fire from corrugation amplitude or corrMGT fallback when corrugation risk is active | 0.20 mm / pass | ~30% per pass | 75% wear rate x 0.85 MGT | High-traffic, low-RCF curves |
| Corrective grinding (tram/metro) | wV >= threshold OR rcf >= threshold; special zones may also trigger on corrugation amplitude or corrMGT fallback | 0.55 mm x N passes (N=1-4) | 18% x N passes | wear factor 0.92; cooldown 2.0 MGT (tram/metro) | Deferred maintenance, high RCF, local special-zone defects |
| Corrective grinding (heavy) | RCF >= band threshold OR MGT interval reached | 0.55 mm x N passes | 18% x N passes | wear factor 0.92; cooldown gi x 0.40 (heavy) | RCF-critical moderate curves |
| Reprofiling | wL >= threshold% of lateral limit (default 60%) | reprRemL (2-4mm L) + reprRemV (0.30 x reprRemL) | Reset 40-60% | 70% wear rate; skip next grind | Tight curves, lateral-wear dominated |
| Tamping (ballasted only) | MGT interval (BASE x f_platform x f_speed) | N/A (ballast, not rail) | N/A | N/A | Settlement control, all contexts |

## A5.1 Key Interaction: Reprofiling Skips Next Grinding

When reprofiling occurs, it restores both the crown (vertical) and gauge face (lateral) profiles. The simulator therefore skips the next scheduled grinding pass (skip-next-grinding toggle, active by default). This avoids double-consuming the vertical reserve when reprofiling and grinding would otherwise coincide.

## A5.2 Reserve is the Binding Lifecycle Constraint

The wear limit (e.g. 11mm for metro vertical) is the absolute dimensional maximum. But the grinding reserve (metal available for removal) may be exhausted BEFORE the wear limit is reached by traffic alone. Example: R350HT with corrective strategy -- 17mm reserve / 2.2mm per intervention = only 7 interventions before reserve exhausted. With preventive strategy -- 17mm / 0.20mm = 85 interventions. This is one of the strongest economic arguments for preventive over corrective grinding.

PART B

Physical Models and Formulas

# 1. Rail Profile -- Geometry and Wear Zones

The rail cross-section defines two independent wear zones: the crown (vertical wear, wV) on the running surface, and the gauge face (lateral wear, wL) on the rail head side. EN 13674-1 governs vignole rail product profiles and steel grades; EN 14811 governs grooved rails and associated construction profiles for tram/special-purpose use. In the simulator, these standards anchor the profile family and section geometry used to define defaults, but they are not treated as direct universal lifecycle action tables.

## 1.1 Profile Families and Simulator Default Limits (EN 13674-1 / EN 14811)

| Profile | Standard | kg/m | Limit V (mm) | Limit L (mm) | Reserve V | Reserve L | Contexts |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 49E1 (S49) | EN 13674-1 | 49.4 | 8 | 9 | 13 mm | 7 mm | Tram, Metro |
| 54E1 (UIC54) | EN 13674-1 | 54.8 | 9 | 11 | 15 mm | 8 mm | Tram, Metro, Heavy |
| 54E5 (AHC) | EN 13674-1 | 54.8 | 9 | 10 | 15 mm | 8 mm | Metro, Heavy |
| 60E1 (UIC60) | EN 13674-1 | 60.2 | 11 | 13 | 17 mm | 9 mm | Metro, Heavy |
| 54G2 (Ri54G2) | EN 14811 | 54.6 | 7 | 8 | 12 mm | 6 mm | Tram, Metro |
| 55G3 (41GP13) | EN 14811 | 55.3 | 7 | 8 | 12 mm | 6 mm | Tram, Metro |
| 59R2 (Ri59N) | EN 14811 | 58.1 | 8 | 9 | 13 mm | 7 mm | Tram, Metro |
| 60R2 (Ri60N) | EN 14811 | 59.8 | 8 | 9 | 13 mm | 7 mm | Tram, Metro |

Note: Metal reserve = total grindable material from new rail to wear limit. Each grinding intervention consumes a fraction of this reserve. The simulator tracks remaining reserve to prevent grinding beyond the physical limit.

## 1.2 Global profile selection and override hierarchy

The simulator applies one global rail profile at project level, filtered by rail type. This profile defines the default vertical limit, lateral limit, vertical reserve, lateral reserve and minimum remaining reserves.

If custom wear limits or custom reserve thresholds are activated, those manual values override the profile defaults without altering the underlying profile identity.

# 2. Traffic and Damage Logic (MGT)

Traffic drives every deterioration mechanism in the simulator. Gross MGT/yr measures line busyness. Equivalent MGT introduces axle-load sensitivity so that a heavier fleet contributes disproportionately more wear and fatigue damage than a lighter fleet at identical gross tonnage.

## 2.1 Formulas

Gross MGT/yr = (Passes/day x Axle load x Bogies x Axles/bogie x 365) / 1,000,000

MGT_eq (wear)  = MGT_gross x (Q_axle / Q_ref)^3   [Archard exponent n=3]

MGT_eq (RCF)   = MGT_gross x (Q_axle / Q_ref)^4   [fatigue exponent n=4]

Reference axle loads: Tram Q_ref=10t, Metro Q_ref=15t, Heavy Q_ref=22.5t. A 20t axle causes ~8x more wear damage than a 10t axle at the same gross tonnage.

Definition of variables used in the MGT formulas:

Passes/day = average train passages per day on one track and one direction, after applying manual, weekly-profile or mileage-profile logic.

Axle load = static axle load in tonnes for the fleet considered.

Bogies = number of bogies per vehicle or trainset representation used in the simulator.

Axles/bogie = number of axles carried by each bogie.

365 = annualisation factor converting daily traffic to yearly traffic.

1,000,000 = conversion from tonnes to million gross tonnes.

MGT_gross = gross million tonnes per year.

MGT_eq = equivalent million tonnes used as a damage proxy rather than as a direct commercial tonnage figure.

Q_axle = actual axle load of the fleet under consideration.

Q_ref = reference axle load for the selected context (tram, metro or heavy).

Exponent 3 = wear-oriented Archard-type sensitivity used in the simulator for rail material removal.

Exponent 4 = fatigue-oriented sensitivity used for RCF weighting, reflecting the stronger nonlinearity of contact-fatigue damage.

## 2.2 Axle Load Sensitivity Table

| Axle load | Ratio to 15t metro ref | Wear factor (n=3) | RCF factor (n=4) | Typical fleet |
| --- | --- | --- | --- | --- |
| 10t (tram) | x0.67 | x0.30 | x0.20 | Urban tramway |
| 12t (light metro) | x0.80 | x0.51 | x0.41 | Light metro / LRT |
| 15t (metro ref) | x1.00 | x1.00 | x1.00 | Reference -- standard metro |
| 17t (heavy metro) | x1.13 | x1.44 | x1.63 | Heavy metro / suburban EMU |
| 20t (regional) | x1.33 | x2.37 | x3.16 | Regional / intercity |
| 25t (freight) | x1.67 | x4.63 | x7.72 | Heavy haul freight |

Key insight: a mixed line where 20% of trains are heavy freight (25t) can have equivalent MGT 2-3x higher than gross MGT alone suggests. Always check equivalent MGT in the simulation results -- it is the operationally relevant number for maintenance planning.

## 2.3 Passes/Day Calculation Modes

| Mode | Formula | When to use |
| --- | --- | --- |
| Manual | Direct trains/day entry | Timetable frequency is known |
| Weekly profile | (5 x weekday + saturday + sunday) / 7 | Significant weekend variation (commuter lines) |
| Mileage profile | (fleet x km/train/yr) / (section_km x 365) | Fleet utilisation data available, timetable unknown |

## 2.4 Multi-Fleet Mixing

MGT_total = Sum_i(Passes_i x AxleLoad_i x Bogies_i x Axles_i x 365 / 1e6)

MGT_eq    = Sum_i(MGT_i x (Q_i / Q_ref)^3)

Note: Tamping retains a gross-tonnage-oriented logic because ballast settlement follows a near-linear damage law (exponent 1-1.5 per Lichtberger 2005), unlike rail fatigue which uses exponent 3-4.

- Archard J.F. (1953) - Contact and Rubbing of Flat Surfaces, J.Applied Physics

https://doi.org/10.1063/1.1721448

- IHHA Wheel-Rail Interface Guidelines 5th ed. (2019)

https://www.ihha.net/

# 3. Radius Bands and Wear Multipliers

Five radius bands define how the wheel-rail contact geometry amplifies wear relative to large-radius / tangent track (r5 = reference). r5 now carries a residual lateral factor of 0.65 (not 1.0) reflecting low but non-zero gauge-face wear observed above 800m. Multipliers are calibrated against the Infrabel/TU Delft 2023 dataset.

## 3.1 Band Definitions, Multipliers and Grinding Intervals

Literature note: large-radius and tangent track are characterised by low gauge-face wear compared with tight curves. This is why the simulator now uses r5 f_L = 0.65 instead of 1.0. The objective is not to force zero lateral wear in tangent track, but to avoid over-penalising large-radius segments and tangent special zones.

Reference interpretation: Belgian big-data wear studies, wheel-rail tribology literature and wear-regime mapping papers all support a strong drop in flange/gauge-face wear as radius increases, even if they do not all propose the same single multiplier.

Metro grinding intervals are intentionally more conservative than the original early presets: r1 is disabled for standard preventive grinding, then r2=18, r3=24, r4=36 and r5=55 MGT. Heavy rail keeps r1 disabled and uses r2=20, r3=30, r4=50 and r5=80 MGT. The rationale is that a purely vertical preventive grind has limited value in extremely tight curves where lateral geometry dominates and reprofiling becomes the more relevant corrective tool.

Representative radius inside a band still matters, but much less than choosing the wrong band itself. The simulator therefore uses a band-based multiplier architecture with a representative radius entry to keep the model practical without pretending to solve continuous contact geometry in full detail.

| Band | Radius | f_V | f_L | Dominant mechanism | Grind metro (MGT) | Grind heavy (MGT) |
| --- | --- | --- | --- | --- | --- | --- |
| r1 | R < 100 m | 6.0 | 15.0 | Flange sliding -- severe gauge face wear | Disabled (999) | Disabled (999) |
| r2 | 100-200 m | 4.0 | 9.0 | Mixed flange + crown contact | 18 | 20 |
| r3 | 200-400 m | 2.5 | 5.0 | Transitional -- both mechanisms active | 24 | 30 |
| r4 | 400-800 m | 1.5 | 2.5 | Crown contact dominant -- RCF peak zone | 36 | 50 |
| r5 (ref) | R >= 800 m | 1.0 | 0.65 | Mostly rolling -- limited gauge-face wear | 55 | 80 |

Tram grinding intervals: r1 disabled (999), r2=12, r3=18, r4=28, r5=40 MGT.

## 3.2 Why Lateral Wear Dominates in Tight Curves

In curves below R200m, the wheel bogie cannot steer freely and the outer wheel flange bears hard against the gauge face. The contact is no longer pure rolling -- it is a combination of rolling and intense lateral sliding, producing wear rates up to 15x higher than tangent track. In contrast, the inner rail in tight curves can experience RCF from the back of the flange, while the outer rail suffers primarily gauge face wear.

## 3.3 Speed Limits by Context (EN 13803 Cant Deficiency)

| Band | Tram Vmax | Metro Vmax | Heavy Vmax |
| --- | --- | --- | --- |
| r1 R<100m | 25 km/h | 35 km/h | 40 km/h |
| r2 100-200m | 35 km/h | 55 km/h | 65 km/h |
| r3 200-400m | 55 km/h | 80 km/h | 95 km/h |
| r4 400-800m | 70 km/h | 100 km/h | 130 km/h |
| r5 >800m | Line speed | Line speed | Line speed |

- Infrabel/TU Delft (2023) - Big-data analysis of rail wear, Wear 522

https://doi.org/10.1016/j.wear.2022.204764

- EN 13803 - Railway applications: Track alignment design parameters

https://standards.cen.eu/

# 4. Wear Rate Model

Base rate: 0.82 mm/100MGT vertical crown wear. R260 grade, tangent, ballasted track, 80 km/h. Source: Infrabel/TU Delft 2023 (5338 km statistical analysis, Belgium 2012-2019).

## 4.1 Formulas

wearRate_V = 0.82 x f_V(band) x he x f_railType x f_trackForm x f_speedV

wearRate_L = 1.00 x 1.5 x f_L(band) x he x f_railType x f_trackForm x f_speedL x f_lubr

he (hardness effect) = 1 - (1 - f_wear_grade) / (1 + f_L x 0.3)  [no upper cap]

The 1.5 base multiplier on wL reflects that lateral (gauge face) wear is structurally 50% more aggressive than vertical wear at reference conditions, due to the sliding contact nature of flange/gauge interaction.

f_lubr applies to lateral wear only. Lubrication reduces flange/gauge friction; the running table remains governed by crown contact and speed effects.

Definition of variables used in the wear-rate formulas:

0.82 = calibrated base vertical wear rate in mm/100MGT for the reference case R260, tangent, ballasted, 80 km/h.

1.00 in wearRate_L = reference lateral base before applying the explicit 1.5 lateral severity multiplier.

1.5 = modelled base amplification of lateral wear relative to crown wear under the reference contact regime.

f_V(band) = vertical wear multiplier associated with the selected radius band.

f_L(band) = lateral wear multiplier associated with the selected radius band.

he = hardness effect factor derived from rail grade and progressively damped in tight curves.

f_wear_grade = grade-dependent wear coefficient from the rail grade table (R200 to R400HT).

f_railType = rail-type multiplier, mainly distinguishing vignole and groove rail environments.

f_trackForm = multiplier associated with ballasted, slab or embedded track form.

f_speedV / f_speedL = speed-dependent multipliers for vertical and lateral wear respectively.

f_lubr = lubrication multiplier applied to lateral wear only.

All wear rates are expressed in mm per 100 MGT equivalent unless explicitly noted otherwise.

## 4.2 Rail Type and Track Form Factors

Literature note: the groove-rail lateral penalty was deliberately reduced from the earlier simulator setting. The current +50% factor is intended as a realistic generic preset for tramway conditions, whereas the former +80% factor behaved more like an extreme-case assumption than a network-average engineering default.

Track-form factors should be read as pragmatic severity multipliers, not as direct structural transfer functions. They capture the operational observation that slab and embedded environments often run harsher on rail than the reference ballasted case.

Grooved rail now uses +20% vertical and +50% lateral wear relative to vignole. This is intentionally lower than the earlier +80% lateral penalty because literature and field return indicate that +80% was too aggressive as a generic preset. The current value is a compromise intended to remain realistic across tramway cases without over-penalising every grooved section.

Track form acts independently from rail type. Ballasted track is the reference, slab adds roughly +10% vertical and +15% lateral severity, and embedded track adds roughly +15% vertical and +20% lateral severity. The simulator treats these as operational severity multipliers rather than as direct structural defect laws.

| Parameter | f_V | f_L | Notes |
| --- | --- | --- | --- |
| Vignole rail (ref) | 1.00 | 1.00 | Reference |
| Groove rail (tram) | 1.20 | 1.50 | Contamination in groove; sustained flange contact |
| Ballasted track (ref) | 1.00 | 1.00 | Reference |
| Slab track | 1.10 | 1.15 | Stiffer substrate, higher dynamic impact |
| Embedded track (tram) | 1.15 | 1.20 | Vibration damping but higher friction environment |

## 4.3 Speed Factors

| Speed range | f_speedV (vertical) | f_speedL (lateral r1) | Basis |
| --- | --- | --- | --- |
| < 40 km/h | 0.92 | 1.05 | Eisenmann: low dynamic load, high angle-of-attack dwell |
| 40-80 km/h (ref) | 1.00 | 1.00 | Reference |
| 80-120 km/h | 1.08 | 0.98 | Increased dynamic loads |
| 120-160 km/h | 1.15 | 0.96 | High-speed dynamic amplification |
| > 160 km/h | 1.25 | 0.94 | TGV/HSL regime |

Note: f_speedL is band-dependent. On r1, lower speeds slightly increase lateral wear (higher angle-of-attack dwell). On r5, speed has negligible effect on lateral wear.

## 4.4 Steel Grade Effects -- Hardness Effect Table

The he formula has no upper cap: R200 (f_wear=1.34) gives he>1 (wears faster than R260). Hard grades give he<1. The benefit of hard grades is strongest on straight track and progressively damped in tight curves.

| Grade | f_wear | f_rcf | wV r5 (straight) | wV r3 (curve) | wV r1 (tight) | RCF vs R260 |
| --- | --- | --- | --- | --- | --- | --- |
| R200 | 1.34 | 1.40 | +29% faster | +20% faster | +9% faster | 140% RCF |
| R260 (ref) | 1.00 | 1.00 | reference | reference | reference | 100% (ref) |
| R320Cr | 0.70 | 0.75 | -25% less | -12% less | -5% less | 75% RCF |
| R350HT | 0.50 | 0.55 | -42% less | -20% less | -9% less | 55% RCF |
| R400HT | 0.38 | 0.40 | -52% less | -25% less | -14% less | 40% RCF |

Note: R200 wears 29% faster than R260 on straight track but only 9% faster in tight curves (r1) -- the geometry dominates. R400HT saves 52% on straight track. Calibrated on Infrabel/TU Delft 2023 + Archard H^-1.2 law.

- Infrabel/TU Delft (2023) - Full paper, Wear 522

https://doi.org/10.1016/j.wear.2022.204764

- Esveld C. (2001) - Modern Railway Track, 2nd ed.

https://www.mrt-productions.nl/

# 5. Rolling Contact Fatigue (RCF) Model

RCF is cyclic plastic deformation at the wheel-rail contact causing surface and sub-surface crack initiation. The RCF index (0 to 1) is a normalised cumulative damage ratio calibrated against the shakedown limit and failure threshold.

## 5.1 RCF Accumulation Formula

RCF_increment/yr = rcfBase x MGT x (1 - min(0.80, wearRate/5.0))

rcfBase = ctx.rcfRate[band] x grade.f_rcf x f_speedV x lubRcfF

The term (1 - min(0.80, wearRate/5.0)) is the crack arrest factor: where wear is intense enough (tight curves), the ratchetted surface is removed before cracks can propagate -- this is the 'magic wear rate' or beneficial wear phenomenon.

Definition of variables used in the RCF formulas:

RCF_increment/yr = annual increase of the normalized RCF index.

rcfBase = pre-wear-modulation fatigue severity coefficient for the segment considered.

ctx.rcfRate[band] = context- and radius-band-specific base RCF coefficient.

grade.f_rcf = rail-grade fatigue multiplier relative to R260.

f_speedV = speed multiplier used in the RCF model; the simulator currently links RCF speed sensitivity to the vertical-speed factor.

lubRcfF = lubrication factor applied to RCF growth; intentionally weaker than the lubrication effect on lateral wear.

MGT = annual traffic load used for the RCF update step.

wearRate/5.0 = simplified crack-arrest term: higher wear removes more damaged surface before cracks can propagate.

min(0.80, ...) = cap on the beneficial-wear effect so that wear never fully suppresses fatigue accumulation in the model.

RCF_after = post-grinding fatigue index after applying the selected number of passes and the current grinding reduction rule.

passes = number of grinding passes executed in the intervention considered.

rcfReduction = per-pass fatigue removal coefficient (preventive or corrective depending on strategy).

## 5.2 RCF Paradox -- Why R400-800m is Most Critical

Tight curves (R<200m): lateral wear is so intense it removes the damaged surface layer faster than RCF can accumulate. This is the magic wear rate (Kapoor 1994) -- the rail self-renews by abrasion.

R400-800m curves: contact forces are sufficient to cause ratchetting plasticity, but NOT sufficient to wear the rail fast enough to remove the damaged layer. Damage accumulates unchecked cycle after cycle. This is why heavy rail grinds r4 for RCF, not for wear.

Tangent track (R>800m): lateral forces are near zero. Ratchetting is limited. RCF progresses slowly.

## 5.3 Ratchetting Mechanism

At each wheel passage, contact pressures of 800-1500 MPa locally exceed the steel yield strength. The surface layer plastically deforms in the rolling/sliding direction by approximately 0.001 mm per passage. This strain accumulates until the material ductility limit is reached and a micro-crack initiates, typically 20-45 degrees below the surface. Grinding removes this ratchetted layer (0.1-0.3mm) before cracks can propagate -- this is the physical justification for preventive grinding.

## 5.4 RCF Index Thresholds and Actions

| Index range | Status | Physical state | Action | Source |
| --- | --- | --- | --- | --- |
| 0.00-0.30 | HEALTHY | Shakedown zone -- elastic shakedown reached, no net plastic deformation | Preventive grinding on interval | Ringsberg 2001, Network Rail |
| 0.30-0.45 | MODERATE | Ratchetting begins, surface cracks 0-1 mm deep, detectable by ultrasound from ~0.45 | Corrective grinding triggered (metro/tram: wV or RCF threshold) | RATP REX 2019, STIB 2021 |
| 0.45-0.65 | HIGH | Cracks 1-5 mm, propagation risk. Heavy rail early-grind zone | Corrective grinding triggered (heavy: band threshold r1=0.45...r3=0.30) | Infrabel REX 2023, IHHA 2019 |
| 0.65-0.70 | CRITICAL | Cracks 5-8 mm, structural risk | Grind only if reserve sufficient, else replace | NR, Infrabel REX |
| >=0.70 (RCF_MAX) | REPLACE | Extensive sub-surface fracture risk | Mandatory replacement -- grinding ineffective | Network Rail, Infrabel REX 2023 |

## 5.5 RCF Reset After Grinding

RCF_after = RCF - passes x rcfReduction x (1 + (1 - RCF) x 0.5)

- Preventive (1 pass): rcfReduction = 0.30 -- removes ~30% of accumulated damage

- Corrective (N passes, max 4): rcfReduction = 0.18 per pass

- Bonus factor (1-RCF)x0.5: deeper relative reduction when RCF is still low (cracks entirely in removed layer)

## 5.6 Heavy Rail Early RCF Triggers (per band)

Interpretation note: these early heavy triggers are calibrated modelling thresholds, not literal values copied from a single standard. They were introduced because the previous heavy model could allow moderate-curve segments to hit replacement RCF without an intermediate corrective grinding opportunity.

The thresholds should therefore be understood as a calibrated intervention gate consistent with heavy-rail RCF practice, literature on moderate-curve fatigue vulnerability, and engineering return from heavy networks, rather than as a universal normative action table.

These early heavy triggers were introduced to prevent moderate-curve segments from reaching RCF_MAX without an intermediate grinding opportunity. In practice, the heavy-rail model now allows grinding when the preventive MGT cycle is reached OR when the segment RCF reaches its band trigger: r1 0.45, r2 0.32, r3 0.30, r4 0.32, r5 0.38.

The trigger is still bounded by reserve feasibility and does not override replacement once RCF reaches 0.70. In other words, it is an earlier intervention gate, not a rescue mechanism for a rail that has already crossed the replacement limit.

| Band | Radius | RCF trigger | Justification |
| --- | --- | --- | --- |
| r1 | R<100m | 0.45 | Wear-dominated regime, higher tolerance before grinding becomes useful |
| r2 | 100-200m | 0.32 | Infrabel REX: corrective threshold on moderate curves |
| r3 | 200-400m | 0.30 | Network Rail NR/L2/TRK/001: corrective trigger at 0.30 |
| r4 | 400-800m | 0.32 | RCF-critical band, slight tolerance vs r3 |
| r5 | R>800m | 0.38 | Slow RCF growth, later trigger acceptable |

- Ringsberg J.W. (2001) - Life prediction of RCF crack initiation, Int.J.Fatigue 23(7)

https://doi.org/10.1016/S0142-1123(01)00011-5

- Infrabel/Int.J.Fatigue (2025) - 212 instrumented curves analysis

https://doi.org/10.1016/j.ijfatigue.2024.108342

# 6. Grinding Strategy -- Triggers, Intervals and Execution

Five distinct trigger conditions are evaluated each annual step. Grinding fires when any condition is met, provided the rail has sufficient reserve and has not reached RCF_MAX. Preventive is MGT-interval based; corrective is condition-based for tram/metro and RCF/MGT hybrid for heavy rail.

For special zones in tram and metro contexts, the corrective trigger uses stricter vertical thresholds than standard segments and a separate rearm logic for wV and RCF. This prevents over-triggering from fVExtra alone while still allowing severe local zones to grind again when the state truly deteriorates.

## 6.1 Five Trigger Conditions

| Trigger | Condition | Applies to | Notes |
| --- | --- | --- | --- |
| grindByMGT | mgtSG >= interval (MGT) | Preventive strategy only | Interval = BANDS[band].grind[context] |
| grindByHeavyRCF | rcf >= HEAVY_RCF_GRIND_TRIGGER[band] | Heavy rail, corrective only | r1=0.45, r2=0.32, r3=0.30, r4=0.32, r5=0.38 |
| grindByCondition | wV >= wVFrac x limitV OR rcf >= threshold | Metro/tram, corrective | Metro: wVFrac=0.30, rcf=0.30 / Tram: 0.35, 0.35 |
| grindByCorrugationAmp | corrAmp >= corrTriggerAmp | Corrugation zones | Amplitude-based; capped at 1 pass if only corrugation trigger |
| grindByCorrugationMGT | mgtSG >= corrMGT | Corrugation zones, corrective | Fallback when amplitude trigger not configured |

## 6.2 Blocking Conditions (grinding does NOT fire if)

- rcf >= RCF_MAX (0.70): too late -- rail must be replaced

- res <= minResV + 0.5 mm: reserve exhausted -- grinding would exceed physical limit

- reprFlag && reprSkip: reprofiling just occurred and skip-next-grinding option is active (avoids double reserve consumption)

## 6.3 Grinding Intervals -- All Contexts (Preventive, MGT)

Literature note: preventive grinding is most defensible on moderate curves and on lines where surface condition control matters. It is intentionally disabled for standard r1 segments in tram, metro and heavy contexts because a crown-only preventive grind adds little value in the tightest curves compared with geometry-driven reprofiling.

Where corrugation risk is active on special zones, the simulator still allows a preventive-like intervention through corrugation amplitude or the legacy corrMGT fallback. This reflects practice better than a pure radius-only preventive schedule.

Preventive grinding on special zones can still be activated through corrugation fallback. When corrugation risk is active, the legacy corrMGT remains available as a fallback interval even though the preferred local trigger is now corrugation amplitude itself. This keeps older engineering practice readable while allowing a more physical roughness-driven trigger.

For standard segments, tram r1, metro r1 and heavy r1 are disabled in preventive grinding because a crown-only preventive pass is considered to have limited value in the tightest curves compared with geometry restoration by reprofiling.

| Band | Tram | Metro | Heavy | Calibration source |
| --- | --- | --- | --- | --- |
| r1 R<100m | Disabled (999) | Disabled (999) | Disabled (999) | Pure grinding has no value in tight curves -- reprofiling required |
| r2 100-200m | 12 MGT | 18 MGT | 20 MGT | RATP 2019, Guangzhou Metro 2022 |
| r3 200-400m | 18 MGT | 24 MGT | 30 MGT | STIB 2021, Singapore LTA 2020 |
| r4 400-800m | 28 MGT | 36 MGT | 50 MGT | Infrabel REX 2023 |
| r5 >=800m | 40 MGT | 55 MGT | 80 MGT | Network Rail NR/L2/TRK/001 |

## 6.4 Grinding Execution Parameters

| Strategy | Passes | Removal/pass | RCF reset/pass | Post-grind wear factor | Post-grind protection |
| --- | --- | --- | --- | --- | --- |
| Preventive | 1 | 0.20 mm | ~30% | 0.75 (-25%) | gi x 0.85 MGT |
| Corrective | ceil(rcf/0.12), max 4 | 0.55 mm | 18% | 0.92 (-8%) | 2.0 MGT (tram/metro) / gi x 0.40 (heavy) |
| Corrugation-only corrective | 1 pass cap | 0.55 mm | 18% | 0.92 | 2.0 MGT (tram/metro) / gi x 0.40 (heavy) |

## 6.5 Reserve Consumption Comparison

| Metric | Preventive | Corrective |
| --- | --- | --- |
| Metal per intervention | 0.20 mm | 2.20 mm (4 passes) |
| Post-grind wear factor | x0.75 (-25%) | x0.92 (-8%) |
| Typical rail life | 400-600 MGT | 200-350 MGT |
| Max interventions R350HT (17mm reserve) | ~85 grindings | ~7 grindings |
| Max interventions R260 (15mm reserve) | ~75 grindings | ~6 grindings |

Note: This reserve comparison is one of the strongest economic arguments for preventive over corrective grinding. Corrective strategy exhausts the reserve in 6-7 interventions; preventive allows 75-85 interventions before replacement.

- Grassie S.L. (2005) - Rail corrugation: measurement, understanding and treatment, Wear 258

https://doi.org/10.1016/j.wear.2004.03.066

- EN 13231-3:2012 - Rail grinding acceptance criteria

https://standards.cen.eu/

# 7. Replacement Criteria

Replacement is triggered the moment ANY ONE of five conditions is met. The simulator checks all conditions at every annual step.

The active wear limits and reserve thresholds come first from the selected rail profile, then from any manual overrides. Replacement is therefore no longer purely context-driven.

## 7.1 Replacement Conditions

Implementation note: the simulator applies the same RCF replacement threshold of 0.70 to all contexts. The contextual differences appear earlier in the chain through wear rate, RCF growth calibration, grinding logic and reserve consumption, not through a different final RCF replacement ceiling.

This same-threshold approach keeps the replacement rule legible while still allowing different contexts to reach that threshold through very different degradation paths.

Important implementation note: the simulator now resolves limits and reserves from the selected rail profile first, then from manual overrides if they are activated. The context tables in this chapter should therefore be read as engineering reference values and defaults, not as the sole source of replacement criteria in the current code.

This matters especially when comparing profiles such as 49E1, 54E1, 54E5 and 60E1, or groove profiles such as 54G2 and 60R2. Different sections can validly reach end-of-life at different dimensional loss even under the same traffic and maintenance strategy.

| Condition | Tram | Metro | Heavy | Standard |
| --- | --- | --- | --- | --- |
| Vertical wear >= limit | 7 mm | 9 mm | 12 mm | Profile-based simulator defaults (EN 13674-1 / EN 14811 + REX) |
| Lateral wear >= limit | 8 mm | 11 mm | 14 mm | Profile-based simulator defaults (EN 13674-1 / EN 14811 + REX) |
| Vertical reserve <= minResV | 3.0 mm | 3.0-3.5 mm | 3.5 mm | Magel 2017, NR NR/L2 |
| Lateral reserve <= minResL | 3.0 mm | 3.0-3.5 mm | 3.5 mm | Infrabel TR 00059, network practice |
| RCF index >= 0.70 (RCF_MAX) | 0.70 | 0.70 | 0.70 | Network Rail, Infrabel REX 2023 |

## 7.2 Metal Reserves by Grade

| Grade | Vertical reserve | Lateral reserve | Min threshold V | Min threshold L |
| --- | --- | --- | --- | --- |
| R200 | 13 mm | 7 mm | 3.0 mm | 3.0 mm |
| R260 | 15 mm | 8 mm | 3.0 mm | 3.0 mm |
| R320Cr | 16 mm | 9 mm | 3.0 mm | 3.0 mm |
| R350HT | 17 mm | 9 mm | 3.5 mm | 3.5 mm |
| R400HT | 18 mm | 10 mm | 4.0 mm | 4.0 mm |

## 7.3 Grinding Reserve vs Wear Reserve

The grinding reserve is the total metal available for removal before the rail cross-section falls below the minimum acceptable profile. This is the ultimate lifecycle constraint.

R350HT example with corrective strategy (2.2 mm per intervention): 17 mm / 2.2 mm = 7.7 -> only 7 interventions before reserve is exhausted, even if the wear limit (17mm total from new to limit) is not reached by traffic alone. With preventive strategy (0.20mm per intervention): 17mm / 0.20mm = 85 interventions -- the rail reaches its wear limit from traffic, not from grinding overconsumption.

- EN 13674-1:2011+A1:2017 - Vignole rail profiles, steel grades and product tolerances (used here as section/profile reference, not as a direct lifecycle wear-limit table)

https://www.en-standard.eu/bs-en-13674-1-2011-railway-applications-track-rail/

- Network Rail NR/L2/TRK/001 - Track inspection and maintenance (2022)

https://www.networkrail.co.uk/

# 8. Flange Lubrication

Lubrication reduces the friction coefficient at the wheel flange / gauge face contact. It acts on lateral wear only (lubF on wL) -- it has no measurable effect on crown wear (wV). A secondary RCF reduction factor (lubRcfF) is also applied, calibrated on Kalousek & Magel 1997 and IHHA 2019.

## 8.1 Lateral Wear Reduction Factors (lubF)

| Mode | r1 R<100m | r2 100-200m | r3 200-400m | r4 400-800m | r5 >800m | Max reduction r1 |
| --- | --- | --- | --- | --- | --- | --- |
| None | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 0% |
| Poor (badly maintained) | 0.80 | 0.83 | 0.90 | 0.97 | 1.00 | -20% |
| Standard (wayside) | 0.45 | 0.52 | 0.68 | 0.92 | 1.00 | -55% |
| Good (wayside + onboard) | 0.28 | 0.35 | 0.55 | 0.88 | 1.00 | -72% |
| Optimal (lab conditions) | 0.20 | 0.25 | 0.45 | 0.82 | 1.00 | -80% |

## 8.2 RCF Reduction Factors (lubRcfF)

Literature note: flange lubrication is primarily a lateral-wear mitigation tool. Its influence on RCF is treated as secondary in the simulator because lubrication reduces damaging flange/gauge creepage, but does not behave like a direct crown-fatigue cure.

This is why the lubRcfF values remain deliberately close to 1.00 compared with the much stronger lateral-wear reduction factors. The simulator reflects a moderated benefit, not a miracle effect.

The lubrication effect on RCF is intentionally weaker than on lateral wear. In the simulator, lubrication is primarily a flange / gauge-face friction control mechanism, not a direct cure for crown-contact fatigue. This is why lubRcfF remains close to 1.00 compared with the much stronger reduction applied to lateral wear.

In large-radius and tangent track, the retained benefit is minimal and the r5 values remain effectively neutral. The strongest RCF moderation appears in the tighter bands where flange-gauge interaction and creepage are sustained for longer.

| Mode | r1 | r2 | r3 | r4 | r5 | Source |
| --- | --- | --- | --- | --- | --- | --- |
| None | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | -- |
| Poor | 0.97 | 0.97 | 0.98 | 0.99 | 1.00 | Marginal effect |
| Standard | 0.90 | 0.88 | 0.90 | 0.96 | 1.00 | Kalousek & Magel 1997 |
| Good | 0.85 | 0.82 | 0.86 | 0.94 | 1.00 | Kalousek & Magel 1997 |
| Optimal | 0.80 | 0.78 | 0.83 | 0.92 | 1.00 | IHHA 2019 |

Note: r5 lubRcfF = 1.00 for all modes: lubricant is not retained in the contact zone on straight track. The effect is strongest on tight curves where flange-rail contact is sustained.

# 9. Reprofiling Strategy

Reprofiling restores the full transversal rail profile -- crown (vertical) AND gauge face (lateral). Unlike grinding which targets the crown only, reprofiling removes metal in both V and L directions.

## 9.1 Trigger and Feasibility

Literature note: geometry-triggered reprofiling is consistent with IHHA, Network Rail and Infrabel practice, where reprofiling is used to restore section geometry when gauge-face deviation or lateral reserve becomes the binding problem.

The lockout for R>=800 m should be understood as a modelling simplification: in the current simulator, reprofiling is reserved for genuinely curvature-driven lateral-geometry cases.

Because reprofiling consumes both vertical and lateral reserve, the feasibility check is one of the most important protections in the simulator. A segment can exceed the geometric threshold for reprofiling and still be refused if the remaining metal would fall below minimum safe reserve after the operation.

This also explains why reprofiling is explicitly locked out for R>=800 m segments and special zones. In those large-radius cases, lateral-geometry restoration is rarely the correct maintenance lever in the current model.

- TRIGGER: Geometry-driven. Initiated when lateral wear reaches the configured threshold (default 60% of lateral limit). Source: IHHA 2019.

- FEASIBILITY: Only triggered if both V and L reserves will remain above minimum thresholds after the operation. Otherwise skipped and replacement follows naturally.

- PRIORITY: If replacement is triggered in the same year, reprofiling is discarded. Replacement takes priority.

- R>=800m: Reprofiling disabled for all segments and special zones with representative radius >= 800m.

## 9.2 Metal Removal per Intervention (Speno TB-2019-04)

| Band | Lateral removal (reprRemL) | Vertical removal (reprRemV = reprRemL x 0.30) |
| --- | --- | --- |
| r1 R<100m | 3.0 mm | 0.90 mm |
| r2 100-200m | 2.0 mm | 0.60 mm |
| r3 200-400m | 1.0 mm | 0.30 mm |
| r4 400-800m | 0.5 mm | 0.15 mm |
| r5 >800m | Disabled | Disabled |

## 9.3 Grinding vs Reprofiling Comparison

| Operation | Axis | Removal/pass | RCF reset | Post-op wear factor | Trigger |
| --- | --- | --- | --- | --- | --- |
| Preventive grinding | Vertical only | 0.10-0.20 mm | Reset 25-30% | 0.75 (-25%) | MGT interval |
| Corrective grinding | Vertical only | 0.30-0.55 mm x N | Reset 18% x N | 0.92 (-8%) | RCF/wV threshold |
| Reprofiling | V and L both | 0.5-1mm V + 2-4mm L | Reset 40-60% | 0.70 (-30%) | Geometry threshold (wL%) |

Note: Skip-next-grinding is active by default after reprofiling. Crown restored by reprofiling -- the next scheduled grinding pass is skipped to avoid double consuming vertical reserve.

- Magel E. et al. (2017) - Wheel-Rail Tribology, Elsevier

https://www.elsevier.com/books/

- Speno International TB-2019-04 - Reprofiling metal removal data

https://www.speno.ch/en/services/rail-grinding/

- IHHA Wheel-Rail Interface Guidelines 5th ed. (2019)

https://www.ihha.net/

# 10. Grinding Stone Consumption

Grinding stones (abrasive wheels) are consumables mounted on each grinding head. Stone wear depends on curve radius, rail grade hardness, and machine type. Stone cost represents 30-50% of own-fleet grinding cost.

## 10.1 Stone Consumption Rates (stones/km/rail/pass)

| Machine | R<100m | R100-200m | R200-400m | R400-800m | Tangent | Stone weight |
| --- | --- | --- | --- | --- | --- | --- |
| Small (tram/metro, 16-24 heads) | 4.0 | 2.5 | 1.5 | 1.0 | 0.8 | 0.9 kg |
| Line machine (32-48 heads) | 6.5 | 3.8 | 2.2 | 1.4 | 1.2 | 1.4 kg |
| Specialist Speno/Loram (64-120 heads) | 10.0 | 6.0 | 3.5 | 2.2 | 2.0 | 2.2 kg |

## 10.2 Grade Hardness Factor on Stone Consumption

| Grade | Stone consumption factor | Effect |
| --- | --- | --- |
| R200 | 0.90 | Soft steel wears stones 10% less than R260 |
| R260 (ref) | 1.00 | Reference |
| R320Cr | 1.15 | +15% stone consumption vs R260 |
| R350HT | 1.30 | +30% |
| R400HT | 1.45 | +45% -- premium grade is hard on abrasives |

Stone consumption formula: Rate (stones/km/pass) = baseRate[machine][band] x gradeFactor[grade] x 2 rails

Definition of variables used in the stone-consumption formula:

baseRate[machine][band] = preset abrasive consumption for the chosen machine family and radius band, per rail and per pass.

gradeFactor[grade] = multiplier reflecting the higher abrasive demand of harder steels.

2 rails = conversion from single-rail rate to total two-rail grinding consumption for the section considered.

Rate is expressed as stones per kilometre per pass in the simulator tables.

## 10.3 Procurement Planning

- First cycle: stone quantity to the first rail replacement. Use for single maintenance contracts.

- Full horizon: total over all replacement cycles. Use for long-term framework contracts, budget planning over 10-30 year concessions, warehouse sizing for remote locations.

Note: Typical stone unit price: 5-20 EUR/stone. The Implied EUR/ml indicator in the simulator helps cross-check your stone unit price against the grinding cost rate.

- Speno International TB-2019-04 - Stone consumption factors by radius

https://www.speno.ch/en/services/rail-grinding/

- Rame I. et al. (2018) - Abrasive wear of grinding wheels in rail grinding, Wear 406-407

https://doi.org/10.1016/j.wear.2018.01.012

# 11. Ballast Tamping Strategy

The tamping module estimates preventive tamping schedules, ballast top-up requirements, degarnissage cycles and lifecycle costs for ballasted track only. It is only active when Track Form = Ballast.

## 11.1 Tamping Interval Formula

Interval (MGT) = BASE[context][band] x f_platform x f_speed

f_speed = sqrt(V_ref / V_segment)   with V_ref = 80 km/h

Higher speed tightens geometric tolerances, requiring more frequent tamping. A 160 km/h segment has f_speed = sqrt(80/160) = 0.71 -> 29% shorter interval than the 80 km/h reference.

Definition of variables used in the tamping interval formulas:

BASE[context][band] = context- and radius-band-specific preventive tamping interval expressed in Gross MGT.

f_platform = platform-quality multiplier representing the settlement sensitivity of the track support.

f_speed = speed multiplier applied to tamping interval; higher speed shortens the interval because geometry tolerances become tighter.

V_ref = 80 km/h reference speed used to normalise the speed effect.

V_segment = representative operating speed of the simulated segment or special zone.

Interval (MGT) = gross traffic accumulated between two tamping interventions in the model.

## 11.2 Base Intervals (Gross MGT) -- REX Calibration

Literature note: using Gross MGT for tamping is deliberate and physically different from rail wear logic. Ballast settlement and geometry retention are much less nonlinear than wheel-rail wear and fatigue, so converting tamping demand through the same equivalent-tonnage law would create unrealistic ballast maintenance intervals.

The current base intervals are therefore best read as pragmatic maintenance calibration values derived from practice and track engineering references, not as direct outputs of a fatigue law.

Special zones inherit their simulated geometry and local speed in the tamping module. They are not assigned a separate ballast law, but they do influence tamping demand through the same radius-band and speed logic as standard segments.

The use of gross MGT rather than equivalent MGT is deliberate here: ballast settlement and geometry retention are much less axle-load nonlinear than rail wear and RCF, so applying the same fatigue-weighted conversion would distort tamping needs.

| Band | Tram | Metro | Heavy | REX source |
| --- | --- | --- | --- | --- |
| r1 R<100m | 8 MGT | 14 MGT | 15 MGT | RATP 2019, STIB 2021, Infrabel 2023 |
| r2 100-200m | 12 MGT | 20 MGT | 22 MGT | RATP 2019, Guangzhou Metro 2022 |
| r3 200-400m | 18 MGT | 27 MGT | 30 MGT | STIB 2021, Singapore LTA 2020 |
| r4 400-800m | 25 MGT | 38 MGT | 40 MGT | Infrabel REX 2023 |
| r5 >=800m | 35 MGT | 48 MGT | 65 MGT | Network Rail NR/L2/TRK/001 |

Note: Tamping uses Gross MGT (not Equivalent MGT). Ballast settlement follows a near-linear damage law (exponent 1-1.5 per Lichtberger 2005), unlike rail fatigue which uses exponent 3-4. Using Equiv.MGT would artificially understate the metro contribution to ballast degradation.

## 11.3 Platform Quality Factors (f_platform)

| Quality | f_platform | Typical soil | Effect on interval | Typical applications |
| --- | --- | --- | --- | --- |
| P1 -- Excellent | 1.20 | Stable granular, gravel, good drainage | +20% longer | Modern embankments, hard rock tunnels |
| P2 -- Good (ref) | 1.00 | Consolidated cohesive clay, stiff marl | Reference | Most WEU main lines |
| P3 -- Fair | 0.70 | Soft clay, silt, moderate drainage | -30% shorter | Older lines, alluvial plains |
| P4 -- Poor | 0.45 | Heterogeneous fill, high water table | -55% shorter | Soft ground, tropical soils, urban metro |

## 11.4 Ballast Degradation and Degarnissage

Ballast degrades cumulatively through three mechanisms: (1) Attrition: each tamping cycle breaks and rounds grain edges (-0.3 to 0.8% volume per cycle); (2) Lateral migration: ballast shifts out of shoulder profile (1-3 kg/m/cycle); (3) Contamination: fines from subgrade, brake dust and crushed grains accumulate.

| Parameter | Value | Range | Source |
| --- | --- | --- | --- |
| Top-up r1 (R<100m) | 50 kg/m / intervention | 30-80 kg/m | Infrabel REX 2023 |
| Top-up r3 (200-400m) | 30 kg/m / intervention | 20-50 kg/m | Network Rail NR/L2/TRK/004 |
| Top-up r5 (>800m) | 15 kg/m / intervention | 10-25 kg/m | Network Rail NR/L2/TRK/004 |
| Degarnissage frequency | Every 6 tamping cycles | 5-7 cycles | ORE B17 |
| Degarnissage top-up factor | 8x standard appoint rate | ORE B17, NR/L2/TRK/004, network practice | UIC 714R, NR/L2/TRK/004 |
| Fines threshold | 30-40% of total ballast mass | -- | EN 13848, ORE B17 |

## 11.5 Own Fleet Cost Formula

opPerMl = (fuelLph x gasoilEurL + maintEurH + labourTeamEurH) / prodMlH

| Parameter | Light 08-16 | Standard 09-3X | Heavy 09-4X | Unit |
| --- | --- | --- | --- | --- |
| Fuel consumption (fuelLph) | 22 | 35 | 55 | L/h |
| Maintenance (maintEurH, WEU) | 120 | 180 | 260 | EUR/h |
| Team cost (labourTeamEurH, WEU) | 48x3=144 | 55x4=220 | 62x5=310 | EUR/h |
| Productivity (prodMlH) | 175 | 300 | 425 | m/h |
| opPerMl WEU own fleet | 1.91 EUR/m | 1.47 EUR/m | 1.50 EUR/m | EUR/ml |

Note: Subcontractor WEU tariffs: Light=8, Standard=11, Heavy=14 EUR/m. Own fleet is 5-7x cheaper but requires capital investment, maintenance staff and operational organisation. The breakeven threshold is approximately 80-100 km/yr of tamping.

Definition of variables used in the tamping own-fleet cost formula:

opPerMl = operating cost per linear meter.

fuelLph = machine fuel consumption in litres per hour.

gasoilEurL = gasoil or diesel cost per litre in the selected region.

maintEurH = maintenance and ownership cost per machine hour.

labourTeamEurH = total labour cost of the tamping team per hour.

prodMlH = delivered productivity in linear meters per hour, including operational losses.

The formula converts hourly cost into cost per meter by dividing the total hourly burden by delivered productivity.

- UIC 714R - Classification of lines for the purpose of track maintenance

https://uic.org/

- ORE B17 - Recommendations for the design of ballasted track (1983)

https://uic.org/

- Network Rail NR/L2/TRK/004 - Ballast specification and management (2019)

https://www.networkrail.co.uk/

# 12. Validation and Calibration

The simulator is calibrated against five reference cases spanning three network types. Predictions use the full engine with current parameters -- no simplified sub-model. The validation panel compares both vertical and lateral wear and reports recommended base wear values back-calculated from each case.

## 12.1 Reference Cases

| ID | Source | Context | Description | Radius | Grade | wV measured | wL measured | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BE1 | Infrabel/TU Delft 2023 | Heavy | Tangent, R260 | R>800m | R260 | 0.82 mm/100MGT | -- | Active |
| BE2 | Infrabel/TU Delft 2023 | Heavy | R500m outer rail | R500m | R260 | 1.40 | 2.80 mm/100MGT | Active |
| BE3 | Infrabel/TU Delft 2023 | Heavy | Tangent R200 steel | R>800m | R200 | 1.10 | -- | Active |
| GZ1 | ScienceDirect Wear 2021 | Metro | Guangzhou Line 1 R300 | R300m | R260 | 2.10 | 6.50 mm/100MGT | Active |
| GZ2 | Railway Sciences 2022 | Heavy | EMU depot R350 | R350m | R260 | -- | -- | Incomparable -- unit mismatch |

GZ2 note: 10.1mm lateral is absolute after 1M passes, not a mm/100MGT rate. Cannot be compared directly to simulator output.

## 12.2 Calibration Support -- Back-Calculation

Method note: the back-calculation is only as good as the representativeness of the reference case. A single measurement from an outlier section should not be used to redefine the whole context. The strongest interpretation is obtained when several comparable measured cases converge on the same correction direction.

In other words, the validation panel should support engineering judgement, not replace it.

Practical interpretation: if Base V recommended is consistently above Base V now across several comparable cases, the current simulator may be underestimating the baseline wear severity for that context. If it is consistently below, the baseline may be too aggressive. The same reasoning applies to Base L.

A single case should not be used blindly as a universal recalibration source. The recommended values are strongest when several cases point in the same direction under comparable geometry, grade, speed, lubrication and profile assumptions.

The validation table shows Base V now / Base V recommended / Delta V% and the same for lateral wear. The recommended base values are back-calculated from each reference case by removing the active case factors (radius, grade, rail type, track form, speed, lubrication). These values are informational only -- they do NOT recalibrate the engine automatically.

These recommended values are advisory only. They are intended to support manual engineering calibration of baseWearV and baseWearL for the current context. They do not recalibrate the simulator automatically.

If a real measurement uses a rail profile different from the current project profile, the case rail profile is used first in the validation prediction. This keeps profile-dependent limits and reserves coherent during comparison.

Meaning of the validation columns:

Base V now / Base L now = the current context base wear values effectively used by the simulator before case-specific multipliers are applied.

Base V recommended / Base L recommended = implied context base values back-calculated from the measured case after removing the active case factors.

Delta V% / Delta L% = percentage difference between the current base and the recommended back-calculated base.

Measured wear = field observation entered by the user or stored in a reference case.

Simulator wear = predicted wear for the same case using the full engine and the case profile when provided.

| Delta % | Status | Recommended action |
| --- | --- | --- |
| < 15% | Green -- Good fit | Use results for budget decisions |
| 15-30% | Yellow -- Review | Check parameters, consider local calibration |
| > 30% | Red -- Recalibrate | Add project-specific case and adjust base wear rates |

## 12.3 Known Limitations

- Annual time step: cannot model intra-year variations in traffic or maintenance sequencing.

- Base wear rates are constant for a given set of inputs, but the effective yearly degradation can still change after grinding, reprofiling and corrugation reset because the simulator applies post-maintenance modifiers and state-dependent triggers.

- No wheel-rail profile evolution: contact geometry assumed constant -- only material loss is tracked.

- No inner/outer rail asymmetry: outer rail is always critical. Inner rail ~30-60% of outer rate.

- No switch and crossing modelling: different wear mechanisms, out of scope.

- Corrugation is a simplified state model: no explicit wavelength spectrum, no RMS roughness law, no vehicle-track dynamic instability solver.

- No seasonal variation: autumn leaf fall (+15-25% wear), winter ice (altered friction mode) not modelled.

- Cost data: rates based on 2022-2023. Live exchange rates via exchangerate-api.com.

- Calibrated on European heavy rail (Belgium) and Chinese metro. Validate locally for other contexts.

- Rooij L. et al. (2023) - Statistical analysis of rail wear, Belgian network, Wear 522

https://doi.org/10.1016/j.wear.2022.204764

- Liu B. et al. (2021) - Rail wear on Guangzhou Metro, Wear 477

https://doi.org/10.1016/j.wear.2021.203830

# 13. Key Standards and References

Reading note: this chapter intentionally mixes four source families used by the simulator: (1) product/profile standards, (2) maintenance and inspection references, (3) scientific literature, and (4) network return-of-experience (REX). These families do not play the same role and should not be read as if they all delivered direct code constants.

Product standards such as EN 13674-1 and EN 14811 are used mainly to anchor rail profile families, steel grades and section geometry. Maintenance documents such as EN 13231-3, UIC 714R or Network Rail standards provide engineering context and process framing. Scientific papers explain mechanisms and trends. REX sources support calibration of practical defaults.

Where the simulator uses a simplified factor, threshold or interval, this should be read as an engineering modelling choice informed by literature and practice, not as a clause-by-clause transcription of any single standard.

The table below therefore states as precisely as possible what each reference contributes: direct geometric framework, maintenance context, physical mechanism, or calibration support.

| Standard / Reference | Title | Usage in simulator |
| --- | --- | --- |
| EN 13674-1:2011+A1:2017 | Railway applications -- Track -- Rail -- Part 1: Vignole railway rails 46 kg/m and above | Product/profile reference for vignole rail families, steel grades and section geometry used to define simulator defaults; not treated as a universal maintenance action table. |
| EN 14811:2019 | Railway applications -- Track -- Special purpose rail -- Grooved rails and associated construction profiles | Product/profile reference for grooved rail families used to define simulator defaults; not treated as a direct lifecycle threshold table. |
| EN 13848-5:2008 | Railway applications -- Track geometry quality -- Part 5: Geometric quality levels -- Plain line | Geometry-quality framework relevant to tamping and track-maintenance philosophy; tamping intervals in the simulator remain calibrated engineering defaults. |
| EN 13803 | Railway applications -- Track -- Track alignment design parameters | Alignment and cant/speed design context used to frame speed sanity checks and context envelopes. |
| EN 13231-3:2012 | Railway applications -- Track -- Acceptance of works -- Rail grinding | Grinding acceptance/process reference used to frame grinding practice; trigger values in the simulator are still model calibrations, not literal clauses from the standard. |
| UIC 712R | Rail defects | Rail-defect terminology and qualitative RCF context; used as defect-language support rather than as the sole source of simulator thresholds. |
| UIC 714R | Classification of lines for the purpose of track maintenance | Maintenance line-classification context; used qualitatively and not as the direct source of ballast top-up factors or degarnissage multipliers. |
| ORE B17 | Recommendations for the design of ballasted track | Ballasted-track engineering context supporting fouling and cleaning-cycle assumptions; the simulator uses a pragmatic default of 6 cycles within the usual 5-7 cycle range. |
| IHHA Guidelines 5th ed. (2019) | Wheel-Rail Interface Guidelines | Mechanism and engineering-range reference for wear, RCF and reprofiling practice; supports modelling ranges rather than providing one-to-one code constants. |
| Network Rail NR/L2/TRK/001 | Track inspection and maintenance | Maintenance-practice reference used as one input for grinding intervals, RCF action philosophy and inspection context. |
| Network Rail NR/L2/TRK/004 | Ballast specification and management | Ballast-management reference supporting top-up and degarnissage engineering assumptions, including the high ballast renewal factor used for cleaning events. |
| Infrabel REX 2023 | Internal technical report (BE network) | Local network calibration source for wear behaviour, grinding practice and tamping intervals. |
| Infrabel/TU Delft (2023) Wear 522 | Big-data analysis of rail wear on 5338 km of Belgian network | Primary quantitative calibration source for the reference vertical wear base and the large-radius / curve trend structure. |
| Ringsberg J.W. (2001) Int. J. Fatigue | Life prediction of rolling contact fatigue crack initiation | Physical/mechanistic support for cumulative RCF damage logic; used qualitatively to structure the simplified RCF accumulation model. |
| Speno TB-2019-04 | Reprofiling metal removal data | Practical reprofiling calibration source for the vertical-to-lateral removal coupling used in the simulator (reprRemV = reprRemL x 0.30). |
| Kalousek & Magel (1997) | Managing the wheel-rail interface | Tribology/interface reference supporting lubrication, wear-mode interpretation and contact-management philosophy; not used as a direct lookup table for every factor. |

End of technical guide